# MAZE SOLVER

A micromouse is an autonomous robot that:

- Enters a maze it has never seen before

- Explores and maps the maze on its own

- Finds the optimal path to a destination cell

- Runs that path as fast as physically possible

Every single component choice, every algorithm decision, every PCB layout rule exists to serve those four goals. Everything about your robot is derived from the maze.

Maze Specifications

Cell size: 20cm × 20cm

This means each unit square of the maze is 200mm × 200mm from wall centre to wall centre.

Wall thickness: approximately 12mm (standard across most competition mazes)

This gives you a clear passage width of:

Clear passage = 200mm - 12mm = 188mm

So the actual open space your robot drives through is 188mm wide.

Maze size: typically 16×16 cells for standard competition, giving a total maze footprint of:

16 × 200mm = 3200mm = 3.2 metres × 3.2 metres

Destination: user-defined cell — not necessarily the centre.

Phase 1 — EXPLORATION :Robot starts at (0,0) Drives through maze Detects walls with sensors Builds internal map Finds destination cell Returns to start (optional but common)

Phase 2 — SPEED RUN Uses built map Plans fastest route Executes at maximum safe speed Reaches destination

Maximum robot width for diagonal moves:

During diagonal traversal, your robot is at 45° to the walls. Its diagonal footprint must fit through the 188mm passage:

Safe diagonal limit = 188mm - 20mm safety margin = 168mm

For a square robot: side × √2 = 168mm

Maximum square side = 118mm

Maximum robot length for turning inside one cell:

During a 90° turn, your robot rotates within the cell. For the robot to stay within its cell during a turn:

Robot length should not exceed clear passage width

Practical limit = 150mm length maximum

These two numbers — 118mm width maximum, 150mm length maximum — are the hard physical boundaries your robot must live within.

in real life:

• Walls are not perfectly aligned
• Maze manufacturing tolerance ±1–2 mm
• Wheel slip
• Sensor error
• PD overshoot
• High-speed oscillations
• Chassis flex
• Vibration

The mouse is a dynamic control system

Have to account for Worst case dynamic deviation

Concept 1 — Degrees of Freedom in a Differential Drive Robot

differential drive — two independently driven wheels, one on each side. This is the standard micromouse configuration. Understanding how it moves is fundamental to understanding why dimensions matter.

Left wheel faster than right → Robot turns right

Right wheel faster than left → Robot turns left

Both wheels same speed → Robot goes straight

Both wheels opposite speed → Robot spins on the spot

The robot rotates around a point called the Instantaneous Centre of Curvature (ICC). When both wheels spin at equal and opposite speeds, the ICC is exactly at the midpoint between the two wheels — this midpoint is called the centre of rotation.

Why does this matter for sizing? Because during a 90° turn in the maze, your robot rotates around this centre point. The space swept during that rotation must fit within the cell. This directly constrains both your track width (distance between wheels) and your robot length.

Concept 2 — The Three Critical Dimensions

Every differential drive robot has three dimensions that define its motion geometry:

Track Width (T) — Distance between the contact points of left and right wheels. Determines turning radius and stability. Wider = more stable but harder to fit in maze. Narrower = tighter turns but tips more easily.

Wheelbase (W) — Distance from wheel axle to the front of the robot (where your sensors are). Affects how the nose swings during turns. Longer nose = nose swings wider during turns.

Overall Length (L) — Total robot length front to back. Must fit within one cell during turning.

Concept 3 — Turn Geometry

When your robot makes a 90° turn at the centre of a cell, it rotates around its centre of rotation. The critical question is: how much space does it sweep?

The worst case is the corner of your robot — whichever point is furthest from the centre of rotation sweeps the largest arc. This swept arc must stay within the cell boundaries.

Maximum swept radius = distance from centre of rotation to furthest robot corner

For a robot of width W_robot and length L_robot, with wheels at the midpoint of length:

Centre of rotation is at the midpoint between wheels

Distance to front corner = √((W_robot/2)² + (L_robot/2)²)

This must be less than the distance from cell centre to wall:

√((W_robot/2)² + (L_robot/2)²) ≤ 94mm (half of 188mm passage)

Constraint 1 — Diagonal Move

Safe limit = 168mm

√(W² + L²) ≤ 168mm

Constraint 2 — Turn Geometry

Robot corners must stay within the cell during a 90° turn:

√((W/2)² + (L/2)²) ≤ 94mm

(W/2)² + (L/2)² ≤ 8836mm²

Constraint 3 — Comfortable Wall Sensing

You want at least 15mm clearance each side between robot body and wall during straight runs:

W ≤ 188mm - (2 × 15mm) = 158mm

But this is a loose upper bound — we'll get a tighter number from the other constraints.

Constraint 4 — Stability

Track width should be at least 50% of robot width to prevent tipping during high-speed cornering. Too narrow a track on a wide robot means it tips outward during turns.

Track width ≥ 0.5 × W_robot

Concept — Centre of Rotation and Turn Sweep

In a differential drive robot, the centre of rotation is always the midpoint between the two wheel contact points. When your robot turns, every part of the robot sweeps an arc around this point.

The turn sweep radius — the maximum distance from centre of rotation to any corner of the robot — is what must fit within the cell. You want to minimise this number.

Turn sweep radius = distance from wheel midpoint to farthest robot corner

If your wheel axle is at position P from the front of a robot of width W and length L:

Distance to front corners = √((W/2)² + P²)

Distance to rear corners  = √((W/2)² + (L-P)²)

Worst case = maximum of these two

To minimise the worst case, set both equal:

P = L - P  →  P = L/2

Mathematically, the axle at the centre of the robot length minimises turn sweep. This is the provable optimal position purely for turning geometry.

But Real Micromice Don't Always Use Centre Placement — Here's Why

Reason 1 — Weight Distribution and Traction

If your battery sits at the centre or rear, placing the wheel axle at the centre keeps the heaviest mass directly over the drive wheels. More weight on drive wheels = more traction = less wheel slip during aggressive acceleration. Wheel slip corrupts your encoder odometry — it's a direct quality-of-navigation issue.

Reason 2 — Sensor Approach Geometry

Your front IR and ToF sensors need to be close to the wall before a turn to accurately detect it. If the axle is at the exact centre, the front sensors are only L/2 from the centre of rotation. Placing the axle slightly behind centre (say 55–60% from front) moves the sensors closer to the wall at turn initiation without dramatically worsening the turn sweep.

Reason 3 — Nose Swing During Turns

With axle at true centre, both the nose and the tail swing equally during turns. With axle slightly behind centre, the nose swings more but the tail swings less — your rear clears walls more easily which matters when exiting a turn into the next cell.

Axle at 55% From Front

For a robot of length L:

Axle position = 0.55 × L from front = 0.45 × L from rear

This is a deliberate slight bias toward rear placement — close enough to centre that turn sweep is nearly optimal, but biased enough to get traction and sensor geometry benefits.

Recalculating Everything With Correct Size

With the component space reality and 55% axle placement, let me derive the new dimensions:

Width Derivation

Minimum width to fit motors side by side with battery and PCB components:

Battery width:          35mm

Motor clearance each side: 12mm

PCB edge margin each side:  8mm

Minimum width = 35 + 12 + 12 + 8 + 8 = 75mm → needs more for routing

Practical minimum = 85mm

Check against diagonal constraint (with 168mm safe limit):

For W = 90mm, L = 110mm:

Diagonal = √(90² + 110²) = √(8100 + 12100) = √20200 = 142mm ✅

Safety margin = 168 - 142 = 26mm — comfortable

Width: 90mm — gives you component space while maintaining generous diagonal clearance.

Length Derivation

Motor + gearbox length (front-to-back): ~35mm including mount

Battery length:                          65mm

Front sensor nose overhang:              20mm minimum

PCB component zones:                     15mm

Overlap between zones (stacked layout):  some components share space

Minimum length = 20 (nose) + 40 (motor zone) + 35 (battery zone) + 15 (rear)

= 110mm

Check turn sweep with axle at 55% from front (60.5mm from front):

Distance to front corners = √((90/2)² + (60.5)²)

= √(2025 + 3660)

= √5685 = 75.4mm ≤ 94mm

Distance to rear corners  = √((90/2)² + (110-60.5)²)

= √(2025 + 2450)

= √4475 = 66.9mm ≤ 94mm

Both corners clear the cell boundary during turns. Front sweep is the worst case at 75.4mm — 18.6mm inside the cell boundary. Safe.

Length: 110mm

| Robot width | 90mm |
| --- | --- |
| Robot length | 110mm |
| Track width | 72mm |
| Axle position from front | 60mm |
| Diagonal footprint | 142mm |
| Diagonal move capable | Yes (26mm margin) |
| 90° turn in cell safe | Yes (18.6mm margin) |
| Component space adequate | Yes |
| Wheels & Drive Geometry What a Wheel Actually Does in a Micromouse Traction — transferring motor torque to the maze floor without slipping. Slip corrupts your encoder odometry directly — if the wheel slips 2mm, your position estimate is wrong by 2mm, and that error accumulates. Odometry — your robot calculates how far it has travelled by counting wheel rotations via the encoder. The accuracy of every position estimate depends on knowing the wheel circumference precisely. Speed ceiling — wheel diameter combined with gear ratio sets your maximum possible speed. You cannot exceed this mechanically. Stability — wheel width and contact patch affect how the robot behaves during aggressive cornering. The Wheel Diameter Equation Everything connects through one equation: Robot Speed = (Motor RPM / Gear Ratio) × π × Wheel Diameter Rearranged to find ideal wheel diameter for a target speed: Wheel Diameter = (Target Speed × Gear Ratio) / (Motor RPM × π) Step 1 — Define target speed (from maze geometry) Step 2 — Choose wheel diameter (from physical constraints) Step 3 — Calculate required gear ratio (from speed + wheel) Step 4 — Pick nearest available ratio (from what Pololu offers) Step 5 — Verify actual speed with that ratio Motor's RPM at Real Operating Voltages Motor: Pololu Micro Metal Gearmotor HP 6V No-load speed at 6V = 30,300 RPM (from Pololu datasheet) RPM scales linearly with voltage: Full charge:8.4V30,300 × (8.4/6) = 42,420 RPMNominal:7.4V30,300 × (7.4/6) = 37,370 RPM Low cutoff:6.4V30,300 × (6.4/6) = 32,320 RPM Design at 7.4V nominal. Always design at nominal, not peak — your mouse must perform consistently as the battery depletes, not just at full charge. Step 1 — Define Your Target Speed From Maze Geometry Your maze has 20cm cells. The most important question is: "How fast can I realistically go given the distances available to accelerate and brake?" This is not a preference — it is a physics calculation. Let's derive it properly. Assume: - Peak acceleration: 8 m/s² (realistic with silicone wheels on melamine) - Peak braking: 10 m/s² (braking harder than accelerating is normal) - You need at least 2 cells of straight to be worth accelerating at all Distance to reach speed V from rest and stop again: Acceleration distance = V² / (2 × 8) = V² / 16 Braking distance = V² / (2 × 10) = V² / 20 Total distance = V²/16 + V²/20 = V²(1/16 + 1/20) = V² × (5 + 4) / 80 = V² × 9/80 = 0.1125 × V² Now calculate for different speeds: Target Speed; Total Distance Needed; Cells Required (20cm each). 1.0 m/s; 0.1125 × 1.0 = 0.113m; 0.56 cells. 1.5 m/s; 0.1125 × 2.25 = 0.253m; 1.3 cells. 2.0 m/s; 0.1125 × 4.0 = 0.450m; 2.3 cells. 2.5 m/s; 0.1125 × 6.25 = 0.703m; 3.5 cells. 3.0 m/s; 0.1125 × 9.0 = 1.013m; 5.1 cells. 3.5 m/s; 0.1125 × 12.25 = 1.378m; 6.9 cells. 4.0 m/s; 0.1125 × 16.0 = 1.800m; 9.0 cells. in practice, maze paths rarely have more than 4–6 consecutive straight cells before a turn. For a practical competitive target, choose a speed that needs 3–5 cells to achieve — fast enough to be competitive, realistic enough to actually achieve in a real maze: Target speed = 2.5 m/s (needs 3.5 cells — achievable in both 8×8 and 16×16) Speed ceiling = 3.0 m/s (needs 5.1 cells — achievable in 16×16, tight in 8×8) Design target: 2.5 m/s. Absolute ceiling: 3.0 m/s. Step 2 — Choose Wheel Diameter From Physical Constraints Now wheel diameter. The constraints are: Constraint A — Ground Clearance Your motor shaft sits at a certain height from the ground. With Pololu Micro Metal motors mounted horizontally on a PCB, the shaft centre is typically 10–12mm above the PCB bottom surface. Add 3mm ground clearance: Minimum wheel radius = 12mm + 3mm = 15mm Minimum wheel diameter = 30mm Constraint B — Robot Height Micromouse walls are 50mm tall in standard mazes. Your robot must fit under them — actually there is no height limit for the robot itself, but keeping it low reduces centre of mass which helps stability during cornering. Typical competitive mice are 25–35mm tall. With a 32mm wheel your axle is 16mm from ground — leaving 9–16mm for PCB and components above axle. Tight but standard. Constraint C — Available Pololu Wheel Sizes Pololu makes wheels that fit their micro metal motor shafts in these diameters: 32mm, 40mm, 60mm, 70mm, 80mm, 90mm. The 32mm is the smallest that fits your ground clearance requirement. Constraint D — Shaft Compatibility Pololu Micro Metal motors have a 1mm D-shaped output shaft. Only wheels with matching 1mm D-bore fit directly. The Pololu #1087 32mm wheel is designed exactly for this. Result: 32mm wheel is your only practical choice from Pololu that satisfies ground clearance and shaft compatibility. Step 3 — Calculate Required Gear Ratio Now you have: - Target speed: 2.5 m/s (design) / 3.0 m/s (ceiling) - Wheel diameter: 32mm - Motor RPM at 7.4V nominal: 37,370 RPM Gear Ratio = (Motor RPM × π × Wheel Diameter) / Target Speed For 2.5 m/s design target: Gear Ratio = (37,370 × π × 0.032) / 2.5 = (37,370 × 0.10053) / 2.5 = 3,757 / 2.5 = 15.0 For 3.0 m/s ceiling: Gear Ratio = (37,370 × π × 0.032) / 3.0 = 3,757 / 3.0 = 12.5 So the mathematically ideal gear ratio range is 12.5:1 to 15:1 for your wheel and speed targets. When you choose a higher gear ratio, three things happen that directly improve controllability: The motor runs slower relative to your robot's speed. At any given robot speed, a higher gear ratio means the motor is spinning at a higher fraction of its RPM range. This gives your DRV8876 more PWM resolution to work with — small PWM changes produce smaller speed changes, making fine control easier. More torque at the wheel. Higher gear ratio multiplies motor torque more. This means your robot responds more crisply to correction commands — when your wall-following algorithm says "turn slightly left," the motor torque response is sharper and more predictable. Wheel slip is less likely. Lower wheel speed at the same robot speed means less centrifugal force throwing the wheel outward during cornering, and more torque-to-inertia ratio keeping the wheel planted. Recalculating for Control-Focused Ratios Available Pololu ratios above 15:1: 20:1, 25:1, 30:1 Speed = (37,370 / Ratio) × π × 0.032 = 3,757 / Ratio 20:1 — The Recommended Compromise At 1.88 m/s nominal, you need exactly 2 cells to reach full speed and stop. This is perfect for your 20cm cell maze because: 2-cell straights are the most common straight length in any maze → You reach full speed on almost every straight → You're always operating at peak efficiency → Control loop has comfortable reaction time At 7.4V with 20:1 and 32mm wheels, your robot reaches its mechanical top speed at 1.88 m/s. You're not firmware-limiting anymore — the gear ratio itself is your speed governor. This is actually ideal for a control-focused build because: - No need for firmware speed limiting logic initially - Motor is always in its natural operating range - Back-EMF provides natural speed regulation - PWM duty cycle maps directly and intuitively to speed At 8.4V (full charge) you get 2.13 m/s — slightly above 7.4V nominal. Your firmware can optionally cap this to 1.88 m/s for consistency, or let it run at 2.13 m/s when battery is fresh. Gear Ratio: 20:1 Top Speed at 7.4V: 1.88 m/s Top Speed at 8.4V: 2.13 m/s Cells to reach & stop: 2.0 cells Motor at target speed: ~100% RPM at nominal Control difficulty: Easy-Medium Upgrade path: Swap to 15:1 or 10:1 when algorithms mature Torque Verification at 20:1 Stall torque of HP 6V at 8.4V: Stall torque at 6V = 1.7 kg·cm Stall torque at 8.4V = 1.7 × (8.4/6) = 2.38 kg·cm After 20:1 gearbox at 75% efficiency = 2.38 × 20 × 0.75 = 35.7 kg·cm Required torque for 8 m/s² acceleration of 100g robot: Force = 0.1 × 8 = 0.8N Torque at wheel = 0.8 × 0.016 = 0.013 N·m = 0.13 kg·cm Available torque is 274× what you need. You are completely torque unconstrained — acceleration is limited purely by traction, which your DRV8876 manages through current limiting. More torque headroom than 15:1 means even crisper response to correction commands. Updated Speed & Gear Summary Revised (20:1). Gear ratio; 15:1; 20:1. Top speed at 7.4V; 2.50 m/s; 1.88 m/s. Top speed at 8.4V; 2.84 m/s; 2.13 m/s. Cells to reach & stop; 3.5 cells; 2.0 cells. Torque headroom; 137×; 274×. Control difficulty; Medium; Easy-Medium. Competitive level; Regional winner; Regional competitive. Upgrade path; Swap to 10:1; Swap to 15:1 then 10:1. Complete Step 3 Locked Parameters Wheel model; Pololu #1087 — 32mm silicone tire. Wheel diameter; 32mm. Gear ratio; 20:1. Motor model; Pololu HP 6V 20:1 (#2368). Top speed at nominal; 1.88 m/s. Top speed at full charge; 2.13 m/s. Distance per encoder count; 0.00614mm (MA732GQ 14-bit). Counts per 20cm cell; 32,573. Max traction-limited accel; 8.8 m/s² (silicone wheels). Step 4 — Motors & Gearbox The Five Fundamental Motor Parameters Parameter 1 — Terminal Resistance (R) — Unit: Ohms This is the electrical resistance of the copper windings. When the motor is stationary (zero RPM), it behaves like a pure resistor. Maximum current flows at this condition: Stall Current = Supply Voltage / Terminal Resistance For Pololu HP 6V motor: Terminal Resistance = 3.9Ω (from datasheet) Stall Current at 8.4V = 8.4 / 3.9 = 2.15A per motor This is the number that tells you the worst-case current your DRV8876 must handle — and the current limit you must set to protect the motor. 2.15A at stall on 8.4V. Your DRV8876 is rated 3.5A — comfortable headroom. Parameter 2 — Back-EMF Constant (Kv) — Unit: RPM/V or V/RPM When a motor spins, it acts as a generator — it produces a voltage opposing the supply voltage. This is called Back Electromotive Force (Back-EMF). The faster it spins, the more back-EMF it generates. Back-EMF Voltage = Kv × Motor RPM The back-EMF constant can be derived from the no-load speed: At no-load, Back-EMF ≈ Supply Voltage (almost all voltage is back-EMF) Kv = No-load RPM / Supply Voltage = 30,300 RPM / 6V = 5,050 RPM/V Why this matters for your control loop: at any given speed, the actual current drawn by the motor is: Current = (Supply Voltage - Back-EMF) / Terminal Resistance = (Supply Voltage - (RPM / Kv)) / R At high speed (high back-EMF), current drops naturally. At low speed or stall, current is maximum. This is why motors draw massive current when starting and less when running — back-EMF is the natural self-limiting mechanism. Your DRV8876 current limit supplements this protection at low speeds. Parameter 3 — Motor Torque Constant (Kt) — Unit: kg·cm/A Torque is directly proportional to current: Motor Torque = Kt × Current Kt is related to Kv through a physical relationship: Kt (in N·m/A) = 1 / (Kv in rad/s/V) For your motor: Kv = 5,050 RPM/V = 5,050 × (2π/60) rad/s/V = 529 rad/s/V Kt = 1/529 = 0.00189 N·m/A = 0.0193 kg·cm/A This tells you something very useful: to command a specific torque, you command a specific current. Your DRV8876's IPROPI current sense output lets you measure actual motor current in real time, which means you can implement torque control — commanding torque directly rather than commanding voltage. This is the foundation of smooth acceleration profiles. Parameter 4 — Mechanical Time Constant (τm) — Unit: milliseconds This is how quickly the motor speed responds to a change in voltage — specifically the time for motor speed to reach 63% of its final value after a step voltage change: τm = (Rotor Inertia × Terminal Resistance) / Kt² For the Pololu HP 6V this is approximately 5–10ms. This number directly tells you the maximum useful bandwidth of your velocity control loop. You cannot command speed changes faster than the motor can physically respond. Setting your control loop faster than 1/τm is pointless — the motor cannot follow. Maximum useful control loop frequency ≈ 1 / τm = 1 / 0.008 = 125 Hz minimum loop rate Practical control loop: 500Hz–1kHz (faster than τm so you catch disturbances before motor fully responds) Parameter 5 — No-Load Current (I₀) — Unit: milliamps Even with no load, the motor draws some current to overcome internal friction and brush resistance: No-load current for Pololu HP 6V = 70mA at 6V At 8.4V approximately = 70 × (8.4/6) = 98mA ≈ 100mA per motor Two motors at no-load: 200mA continuous just for motors spinning freely. Important for battery life calculations. Your Specific Motor — Pololu HP 6V Full Specification Full Part Name: Pololu Micro Metal Gearmotor HP 6V 20:1 Pololu Item Number: #2368 Available on Robu.in: Search "Pololu 2368" or "Pololu micro metal HP 20:1" Parameter; Value; Significance. Rated voltage; 6V; Can run up to 9V with current limiting. Terminal resistance; 3.9Ω; Sets stall current. No-load speed at 6V; 30,300 RPM; Scales linearly with voltage. No-load speed at 7.4V; 37,370 RPM; Your operating point. No-load speed at 8.4V; 42,420 RPM; Full charge. No-load current at 6V; 70mA; Idle power draw. Stall torque at 6V; 1.7 kg·cm; Before gearbox. Stall current at 6V; 1.6A; Before current limiting. Stall current at 8.4V; 2.15A; Set DRV8876 limit below this. Gear ratio; 20:1; Your chosen ratio. Gearbox efficiency; ~75%; Energy lost in gear mesh. Output stall torque; 25.5 kg·cm; After 20:1 gearbox at 75%. Output shaft speed at 7.4V; 1,868 RPM; After 20:1 gearbox. Motor body diameter; 10mm. Motor body length; 24mm; Including gearbox. Shaft diameter; 1mm D-shape; Matches Pololu #1087 wheel. Why HP Variant Specifically — Not the Standard or MP Variant Pololu makes three power variants of the Micro Metal motor: Variant; Terminal Resistance; Stall Torque; Speed; Best For. Standard; High (~20Ω); Low; Low; Slow, low power robots. MP (Medium Power); Medium (~10Ω); Medium; Medium; General purpose. HP (High Power); Low (3.9Ω); High (1.7 kg·cm); High; High speed, responsive. LP (Low Power); Very high; Very low; Very low; Battery-sensitive slow robots. The HP variant has the lowest terminal resistance of the series. Lower resistance means: Higher stall current → Higher torque capability Faster electrical time constant → More responsive to control commands Better back-EMF generation → More natural speed regulation For a high-speed mouse with closed-loop current control via DRV8876, the HP variant is the only sensible choice. The higher stall current is managed by the driver — you get all the torque and response benefits without the thermal risk. The Gearbox — What 20:1 Actually Means Inside The gearbox attached to your motor is a planetary gearbox — not a simple spur gear train. Understanding the difference matters: Spur gear train — gears in a line, each meshing with the next. Each stage adds backlash. Efficiency drops significantly with more stages. Planetary gearbox — a central sun gear, surrounded by planet gears, enclosed by a ring gear. Multiple teeth in mesh simultaneously. Higher efficiency, lower backlash, more compact. ====================================================== Step 4 — Motors & Gearbox The motor is the component that converts electrical energy from your battery into physical motion Straight run: Both motors spin at equal speed — robot goes straight Slight correction: One motor slows fractionally — robot nudges left or right 90° turn: One motor reverses, other continues — robot pivots Arc transition: Both motors run at slightly different speeds smoothly Diagonal move: Both motors run at specific ratio maintaining 45° heading Emergency stop: Both motors reverse hard — robot decelerates Every single one of these actions requires the motor to respond precisely, quickly, and repeatably to commands from your STM32. If the motor responds sluggishly, your wall corrections arrive late. If it responds inconsistently, your turns overshoot. If it overheats, your later runs are slower than your first run. Step 4.1 — Types of Motors here are four motor types you will encounter when researching micromouse: |  |